# AH Campus 2 Branch Network Configuration Report

## 1. Project Overview

The AH Campus 2 Branch Network supports staff and student lab users through VLAN-based segmentation. Router-on-a-Stick inter-VLAN routing is implemented, with DHCP services provided by the branch router and RIPv2 enabling WAN connectivity.

## 2. Network Topology Diagram

## 3. Address Table

| VLAN ID | Network | Network Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| 90 | Staff | 192.168.9.0 | 255.255.255.0 | 192.168.9.1 |
| 100 | Student Lab | 192.168.10.0 | 255.255.255.0 | 192.168.10.1 |

## 4. VLAN and Port Assignment Table

| VLAN | VLAN Name | Router Interface | Purpose |
| --- | --- | --- | --- |
| 90 | STAFF | G0/0.90 | Staff Users |
| 100 | STUDENT-LAB | G0/0.100 | Student Lab PCs |

## 5. Switch Configuration

hostname Campus2-SW1
!
vlan 90
 name STAFF
!
vlan 100
 name STUDENT-LAB
!
interface range fa0/1-12
 switchport mode access
 switchport access vlan 90
!
interface range fa0/13-24
 switchport mode access
 switchport access vlan 100
!
interface g0/1
 switchport mode trunk
!
spanning-tree mode pvst
!
end

## 6. Router Configuration

hostname Router – Allen H Campus 2 Branch Network
!
ip dhcp pool Staf-pool
 network 192.168.9.0 255.255.255.0
 default-router 192.168.9.1
 dns-server 192.168.9.1
 domain-name wr
!
ip dhcp pool Studlb-pool
 network 192.168.10.0 255.255.255.0
 default-router 192.168.10.1
 dns-server 192.168.10.1
!
interface GigabitEthernet0/0
 no ip address
!
interface GigabitEthernet0/0.90
 encapsulation dot1Q 90
 ip address 192.168.9.1 255.255.255.0
!
interface GigabitEthernet0/0.100
 encapsulation dot1Q 100
 ip address 192.168.10.1 255.255.255.0
!
interface Serial0/2/0
 ip address 10.10.10.2 255.255.255.252
!
router rip
 version 2
 network 10.0.0.0
 network 192.168.9.0
 network 192.168.10.0
!
end