# AH Main Campus Network Configuration Report

## 1. Project Overview

The AH Main Campus network uses VLAN-based segmentation with Router-on-a-Stick inter-VLAN routing. Each department is assigned a unique VLAN and subnet. DHCP services are provided by the main router, and RIPv2 enables WAN routing.

## 2. Network Topology Diagram

## 3. Address Table

| VLAN | Department | Network | Subnet Mask | Gateway |
| --- | --- | --- | --- | --- |
| 10 | Admin | 192.168.1.0 | 255.255.255.0 | 192.168.1.1 |
| 20 | HR | 192.168.2.0 | 255.255.255.0 | 192.168.2.1 |
| 30 | Finance | 192.168.3.0 | 255.255.255.0 | 192.168.3.1 |
| 40 | Business | 192.168.4.0 | 255.255.255.0 | 192.168.4.1 |
| 50 | ENG/CS | 192.168.5.0 | 255.255.255.0 | 192.168.5.1 |
| 60 | Art & Design | 192.168.6.0 | 255.255.255.0 | 192.168.6.1 |
| 70 | Student Lab | 192.168.7.0 | 255.255.255.0 | 192.168.7.1 |
| 80 | IT | 192.168.8.0 | 255.255.255.0 | 192.168.8.1 |

## 4. Switch Configuration (Sample Access Switch)

hostname Switch-Admin
!
vlan 10
 name ADMIN
!
interface range fa0/1-24
 switchport mode access
 switchport access vlan 10
!
interface g0/1
 switchport mode trunk
!
end

Note: Additional switches follow the same configuration pattern,
changing only the VLAN ID and VLAN name to match their department.